# Floods

# R1 — ISP Router

configure

set interfaces ethernet eth0 address 203.0.113.1/30

set system host-name R1-ISP

commit

save

# R2 — Edge Router

configure

set interfaces ethernet eth0 address 203.0.113.2/30

set interfaces ethernet eth1 address 10.255.1.1/30

set system host-name R2-EDGE

set protocols static route 0.0.0.0/0 next-hop 203.0.113.1

set protocols ospf area 0 network 10.255.1.0/30

commit

save

# R3 — Core Router

configure

set interfaces ethernet eth0 address 10.255.1.2/30

set interfaces ethernet eth1 address 10.255.2.1/30

set interfaces ethernet eth2 address 10.255.3.1/30

set interfaces ethernet eth3 address 10.255.4.1/30

set interfaces ethernet eth4 address 10.255.5.1/30

set interfaces ethernet eth5 address 10.0.0.2/24

set system host-name R3-CORE

set protocols ospf area 0 network 10.255.1.0/30

set protocols ospf area 0 network 10.255.2.0/30

set protocols ospf area 0 network 10.255.3.0/30

set protocols ospf area 0 network 10.255.4.0/30

set protocols ospf area 0 network 10.255.5.0/30

set protocols ospf area 0 network 10.0.0.0/24

commit

save

# R4 — Hospital Router

configure

set interfaces ethernet eth0 address 10.255.2.2/30

set interfaces ethernet eth1 address 10.10.1.1/24

set system host-name R4-HOSPITAL

set protocols ospf area 0 network 10.255.2.0/30

set protocols ospf area 0 network 10.10.1.0/24

commit

save

# R5 — Emergency Router

configure

set interfaces ethernet eth0 address 10.255.3.2/30

set interfaces ethernet eth1 address 10.20.1.1/24

set system host-name R5-EMERGENCY

set protocols ospf area 0 network 10.255.3.0/30

set protocols ospf area 0 network 10.20.1.0/24

commit

save

# R6 — University Router

configure

set interfaces ethernet eth0 address 10.255.4.2/30

set interfaces ethernet eth1 address 10.30.1.1/24

set system host-name R6-UNIVERSITY

set protocols ospf area 0 network 10.255.4.0/30

set protocols ospf area 0 network 10.30.1.0/24

commit

save

# R7 — Municipality Router

configure

set interfaces ethernet eth0 address 10.255.5.2/30

set interfaces ethernet eth1 address 10.40.1.1/24

set system host-name R7-MUNICIPAL

set protocols ospf area 0 network 10.255.5.0/30

set protocols ospf area 0 network 10.40.1.0/24

commit

save

# Default Route toward Internet (Run on R3)

configure

set protocols static route 0.0.0.0/0 next-hop 10.255.1.1

commit

save

# ✅ VPCS HOST COMMANDS

## Hospital

ip 10.10.1.10/24 10.10.1.1

ip 10.10.1.11/24 10.10.1.1

ip 10.10.1.12/24 10.10.1.1

ip 10.10.1.13/24 10.10.1.1

## Emergency

ip 10.20.1.10/24 10.20.1.1

ip 10.20.1.11/24 10.20.1.1

ip 10.20.1.12/24 10.20.1.1

ip 10.20.1.13/24 10.20.1.1

## University

ip 10.30.1.10/24 10.30.1.1

ip 10.30.1.11/24 10.30.1.1

ip 10.30.1.12/24 10.30.1.1

ip 10.30.1.13/24 10.30.1.1

## Municipality

ip 10.40.1.10/24 10.40.1.1

ip 10.40.1.11/24 10.40.1.1

ip 10.40.1.12/24 10.40.1.1

ip 10.40.1.13/24 10.40.1.1

# Quick Verification Commands

Run anywhere:

show ip ospf neighbor show ip route ping 10.20.1.10 ping 8.8.8.8

# Earthquake

# pfSense (Earthquake)

### Interfaces

WAN

IP: 198.51.100.2/30

GW: 198.51.100.1

LAN

IP: 10.0.0.1/24

# R1 — ISP (VyOS/Cisco equivalent)

configure

set interfaces ethernet eth0 address 198.51.100.1/30

set system host-name R1-LTE

commit

save

# R2 — Edge Router

configure

set interfaces ethernet eth0 address 198.51.100.2/30

set interfaces ethernet eth1 address 10.255.1.1/30

set interfaces ethernet eth2 address 10.255.6.1/30

set system host-name R2-EDGE

set protocols ospf area 0 network 10.255.1.0/30

set protocols ospf area 0 network 10.255.6.0/30

commit

save

# R3 — Core Router (Mesh Hub)

configure

set interfaces ethernet eth0 address 10.255.1.2/30

set interfaces ethernet eth1 address 10.255.2.1/30

set interfaces ethernet eth2 address 10.255.3.1/30

set interfaces ethernet eth3 address 10.255.4.1/30

set interfaces ethernet eth4 address 10.255.5.1/30

set interfaces ethernet eth5 address 10.0.0.2/24

set system host-name R3-CORE

set protocols ospf area 0 network 10.255.1.0/30

set protocols ospf area 0 network 10.255.2.0/30

set protocols ospf area 0 network 10.255.3.0/30

set protocols ospf area 0 network 10.255.4.0/30

set protocols ospf area 0 network 10.255.5.0/30

set protocols ospf area 0 network 10.0.0.0/24

commit

save

# R4 — Backup Router

configure

set interfaces ethernet eth0 address 10.255.6.2/30

set system host-name R4-BACKUP

set protocols ospf area 0 network 10.255.6.0/30

commit

save

# R5 — Hospital

configure

set interfaces ethernet eth0 address 10.255.2.2/30

set interfaces ethernet eth1 address 10.10.1.1/24

set system host-name R5-HOSPITAL

set protocols ospf area 0 network 10.255.2.0/30

set protocols ospf area 0 network 10.10.1.0/24

commit

save

# R6 — Emergency

configure

set interfaces ethernet eth0 address 10.255.3.2/30

set interfaces ethernet eth1 address 10.20.1.1/24

set system host-name R6-EMERGENCY

set protocols ospf area 0 network 10.255.3.0/30

set protocols ospf area 0 network 10.20.1.0/24

commit

save

# R7 — University

configure

set interfaces ethernet eth0 address 10.255.4.2/30

set interfaces ethernet eth1 address 10.30.1.1/24

set system host-name R7-UNIVERSITY

set protocols ospf area 0 network 10.255.4.0/30

set protocols ospf area 0 network 10.30.1.0/24

commit

save

# R8 — Office

configure

set interfaces ethernet eth0 address 10.255.5.2/30

set interfaces ethernet eth1 address 10.40.1.1/24

set system host-name R8-OFFICE

set protocols ospf area 0 network 10.255.5.0/30

set protocols ospf area 0 network 10.40.1.0/24

commit

save

# VPCS Commands

## Hospital

ip 10.10.1.10/24 10.10.1.1

ip 10.10.1.11/24 10.10.1.1

ip 10.10.1.12/24 10.10.1.1

ip 10.10.1.13/24 10.10.1.1

## Emergency

ip 10.20.1.10/24 10.20.1.1

ip 10.20.1.11/24 10.20.1.1

ip 10.20.1.12/24 10.20.1.1

ip 10.20.1.13/24 10.20.1.1

## University

ip 10.30.1.10/24 10.30.1.1

ip 10.30.1.11/24 10.30.1.1

ip 10.30.1.12/24 10.30.1.1

ip 10.30.1.13/24 10.30.1.1

## Office

ip 10.40.1.10/24 10.40.1.1

ip 10.40.1.11/24 10.40.1.1

ip 10.40.1.12/24 10.40.1.1

ip 10.40.1.13/24 10.40.1.1

# Internet Route (Run on Core)

configure set protocols static route 0.0.0.0/0 next-hop 10.255.1.1 commit save