## Floods

## GNS3 IMPLEMENTATION FOR FLOOD DISASTER SCENARIO:

(Using pfSense Firewall and VyOS Routers)

## 1 Simulation Environment
Tools

- GNS3 (with GNS3VM)

- pfSense Community Edition (Firewall VM)

- VyOS 1.4 (Router VM)

- VPCS / Linux hosts (end devices)

### Hardware Assumptions

- Partial power availability

- Upper-floor infrastructure survives

- LTE backup network remains operational

## 2 Flood Scenario Network Topology (Logical)

- ISP (Primary)

- LTE / Backup ISP

- pfSense Firewall (Command Center)

- VyOS Routers:

  - Hospital

  - Emergency Services

  - Command Center

- End devices:

  - Laptops

  - Mobile devices

  - Limited wired PCs

(Topology diagram referenced earlier)

## 3 pfSense Configuration (Flood)

### Interfaces

| Interface | IP Address |
| --- | --- |
| WAN (ISP) | 203.0.113.2/30 |
| WAN2 (LTE) | 198.51.100.2/30 |
| LAN | 10.0.0.1/24 |

### Key Features Enabled

- Multi-WAN failover

- Stateful firewall

- NAT for all internal subnets

### Flood-Specific Behavior

- Primary ISP degraded

- LTE becomes active backup

- Emergency traffic prioritized

## 4 VyOS Configuration (Flood)

### Interface Assignment (Hospital Example)

configure

set interfaces etherneteth0 address 10.0.0.2/24

set interfaces etherneteth1 address 10.10.1.1/24

set interfaces etherneteth2 address 10.20.1.1/24

set system host-name VyOS-Hospital

commit

save

### Dynamic Routing (OSPF)

configure

set protocols ospf area 0 network 10.0.0.0/24

set protocols ospf area 0 network 10.10.0.0/16

set protocols ospf area 0 network 10.20.0.0/16

commit

save

Automatically reroutes traffic around flooded segments

## 5 Flood Simulation in GNS3

### Failure Injection

- Ground-floor switches powered off

- Several wired links disconnected

- Some desktop PCs removed

- Primary ISP link degraded

### Surviving Elements

- pfSenseVM

- VyOS routers

- LTE backup

- Mobile and laptop nodes

## 6.9.17 Testing (Floods)

## 1 Connectivity

### Purpose

To verify whether end-to-end communication is restored between critical nodes after the disaster.

### Test Method

ICMP echo requests between hosts.

### Command

ping 10.10.1.10

### Observations Recorded

- Reachability (Success / Failure)

- Average RTT

- Packet loss percentage

Confirms basic network availability

Figure 11: ICMP echo requests between hosts

## 2 Failover (WAN / Path Resilience)

### Purpose

To evaluate network behavior when the primary link fails.

### Test Method

Manual shutdown of primary ISP interface in pfSense.

### Procedure

- Disable ISP interface in pfSense

- Observe automatic switch to LTE / backup link

### Verification

- pfSense gateway status

- Continued connectivity from internal hosts

### Command (optional verification)

ping 8.8.8.8

### Observations Recorded

- Failover success

- Time taken to switch links

Confirms resilience against link failure

Figure 12: WAN Failover and Path Resilience Test

## 3 Routing Stability

### Purpose

To assess routing consistency and convergence after topology changes.

### Test Method

OSPF neighbor monitoring on VyOS routers.

### Command

showipospf neighbor

### Observations Recorded

- Neighbor state (Full / Down)

- Route convergence behavior

- Route re-advertisement after failures

Confirms dynamic routing reliability

Figure 13: OSPF Routing Stability Test Results

## 4 Latency

### Purpose

To measure delay introduced by disaster recovery paths and virtualized routing.

### Test Method

Repeated ICMP echo measurements.

### Command

ping -c 20 10.10.1.10

### Observations Recorded

- Minimum RTT

- Average RTT

- Maximum RTT

Indicates network performance degradation

Figure 14: ICMP Echo Latency Measurement

## 5 Packet Loss

### Purpose

To measure data loss due to congestion, unstable links, or degraded infrastructure.

### Test Method

UDP traffic generation using iperf.

### Command

iperf -c 10.10.1.10 -u -b 20M

### Observations Recorded

- Packet loss percentage

- Jitter

- Throughput

Validates network reliability under load

Figure 15: Packet Loss Measurement Using iPerf (UDP)

## 6 Recovery Time

### Purpose

To measure how quickly communication is restored after failures.

### Test Method

Manual timing of service restoration.

### Procedure

- Introduce failure (link down / node removal)

- Measure time until:

  - Ping succeeds

  - OSPF neighbors reach FULL state

### Tools

- GNS3 event timestamps

- Stopwatch / system clock

### Observations Recorded

- Time to first connectivity

- Time to routing convergence

Demonstrates speed of disaster recovery

Figure 16: Network Recovery Time Test Results

Earthquake

## GNS3 IMPLEMENTATION FOR EARTHQUAKE DISASTER SCENARIO

(Using pfSense Firewall and VyOS Routers):

## 1 Simulation Environment

### Tools

### GNS3

- pfSense Firewall VM

- VyOS Router VM

- Mobile & laptop nodes

### Earthquake Assumptions

- Sudden, widespread infrastructure collapse

- Core routers and switches destroyed

- Power grid failure

## 2 Earthquake Scenario Network Topology (Logical)

- LTE / Mobile ISP only

- pfSense Firewall (Emergency Command Center)

- VyOS Routers in flat mesh

- Mobile and temporary devices

(Topology diagram referenced earlier)

## 3 pfSense Configuration (Earthquake)

### Interfaces

| Interface | IP Address |
| --- | --- |
| WAN (LTE) | 198.51.100.2/30 |
| LAN | 10.0.0.1/24 |

### Earthquake-Specific Setup

- Single WAN operation

- Strict firewall rules

- VPN enabled for emergency teams

## 4 VyOS Configuration (Earthquake)

### Interface Assignment

configure

set interfaces etherneteth0 address 10.0.0.3/24

set interfaces etherneteth1 address 10.30.1.1/24

set system host-name VyOS-Emergency

commit

save

### Flat Mesh Routing (OSPF)

configure

set protocols ospf area 0 network 10.0.0.0/24

set protocols ospf area 0 network 10.30.0.0/16

commit

save

✔ No hierarchy — adaptive routing only

## 5 Earthquake Simulation in GNS3

### Failure Injection

- All switches removed

- Core router removed

- Multiple links disabled simultaneously

### Surviving Elements

- pfSense on generator

- VyOS on laptops

- LTE connectivity

- Mobile devices

6.11.18 Testing (Earthquake)

## 1 Connectivity

### Purpose

To verify whether basic communication is restored between surviving nodes.

### Test Method

ICMP echo between emergency nodes.

### Command

ping 10.30.1.10

### Observations Recorded

- Reachability (Yes / No)

- Packet loss

- Average RTT

Confirms network availability after sudden failure

Figure 19: Connectivity Test Results Using ICMP Echo

## 2 Routing Convergence

### Purpose

To evaluate how quickly the network re-establishes routes after simultaneous failures.

### Test Method

OSPF neighbor monitoring on VyOS routers.

### Command

showipospf neighbor

### Observations Recorded

- Neighbor state transitions

- Time to reach FULL state

- Route stability after convergence

Confirms fast adaptation in flat emergency topology

Figure 20: Routing Convergence Test Results (OSPF)

## 3 Latency

### Purpose

To measure delay caused by LTE-only backhaul and emergency routing paths.

### Test Method

Repeated ICMP measurements.

### Command

ping -c 10 8.8.8.8

### Observations Recorded

- Minimum RTT

- Average RTT

- Maximum RTT

Indicates performance degradation due to constrained links

Figure 21: ICMP Latency Test Results

## 4 Packet Loss

### Purpose

To quantify traffic loss caused by unstable wireless links and congestion.

### Test Method

UDP traffic generation.

### Command

iperf -c 10.30.1.10 -u -b 10M

### Observations Recorded

- Packet loss percentage

- Jitter

- Effective throughput

Validates network reliability under stress

Figure 22: Packet Loss Test Results (iPerfUDP)

## 5 Recovery Time

### Purpose

To measure how quickly the emergency network becomes operational after failure.

### Test Method

Timed restoration after failure injection.

### Procedure

- Simultaneously disable multiple links/nodes

- Measure time until:

  - Ping succeeds

  - OSPF neighbors stabilize

### Observations Recorded

- Time to first connectivity

- Time to full routing convergence

Demonstrates rapid emergency network deployment

Figure 23: Emergency Network Recovery Time Results